Fastest Line Follower Bot
Technical Documentation

ESP32 + TB6612FNG + QTR Sensors + Bluetooth PID Tuning

| Project Name | Fastest Line Follower Bot |
| --- | --- |
| Controller | ESP32 (38-pin variant) |
| Drive System | TB6612FNG dual H-bridge + N20 DC motors |
| Sensing | 10-element QTR reflectance sensor array |

# 1. Project Overview

This project is a high-speed autonomous line-following robot built around the ESP32. The bot uses a 10-sensor QTR array to detect the track, a PID control loop to stay centered, and a TB6612FNG motor driver to regulate two N20 DC motors. Bluetooth communication is included for real-time tuning of control parameters without reprogramming the board.

# 2. Objectives

Detect the line position accurately using a wide QTR sensor array.

Move the robot at high speed while keeping stable tracking.

Use PID control to correct errors smoothly and quickly.

Allow live tuning of PID and speed parameters through Bluetooth.

Provide a practical hardware and firmware base for race-style line follower robots.

# 3. Hardware Used

ESP32 (38-pin variant): Main controller for sensor reading, PID computation, motor output, and Bluetooth communication.

TB6612FNG Motor Driver: Dual H-bridge driver used to control two brushed DC motors with PWM speed control.

QTR Sensor Array: 10-sensor reflectance array for line detection and position estimation.

N20 DC Motors (3000 RPM): Compact high-speed motors used for responsive movement and sharp turns.

Buck Converter: Steps down the battery voltage to a stable supply for the electronics.

Power Source: Typically a 12 V battery or equivalent source for the full robot system.

# 4. Software and Libraries

QTRSensors library for reading and calibrating the line sensor array.

SparkFun_TB6612 library for motor control through the TB6612FNG driver.

BluetoothSerial library for wireless command input from a phone or PC.

# 5. Working Principle

The QTR sensors read the reflectance pattern of the track and estimate the line position.

The robot compares the measured position with the center position (4500).

The error is fed into a PID controller to compute the correction value.

The correction adjusts the right and left motor speeds in opposite directions.

Bluetooth commands can update Kp, Ki, Kd, base speed, and maximum speed while the robot is running.

# 6. Firmware Architecture

Global Configuration: Defines motor pins, button pins, sensor count, PID variables, and speed limits.

Motor Control Functions: set_motor_speed() and mspeed() apply speed commands to the TB6612FNG outputs.

PID Loop: pid_control() reads the sensor array, computes P/I/D terms, and updates motor speeds.

Calibration Routine: setup() moves the robot in both directions while calling qtr.calibrate() to learn sensor ranges.

Bluetooth Command Parser: bluetooth_command_handler() receives text commands and updates tuning values in real time.

# 7. Pin Mapping

| Signal | ESP32 Pin | Purpose |
| --- | --- | --- |
| RIN2 | 22 | Right motor direction pin 1 |
| RIN1 | 21 | Right motor direction pin 2 |
| RPWM | 23 | Right motor PWM pin |
| LIN2 | 33 | Left motor direction pin 1 |
| LIN1 | 25 | Left motor direction pin 2 |
| LPWM | 32 | Left motor PWM pin |
| RBUT | 35 | Right button input |
| LBUT | 34 | Left button input |

The QTR sensor inputs are connected to: 14, 13, 27, 26, 5, 17, 16, 4, 19, and 18.

# 8. PID Control Details

The PID controller uses the line position from qtr.readLineBlack() and compares it with the center position of 4500. The error is computed as center_position - position. The three terms behave as follows:

P term: immediate correction based on current error.

I term: accumulates error over time to remove small steady offsets.

D term: reacts to the rate of change of error to reduce overshoot and oscillation.

# 9. Bluetooth Commands

| Command | Parameter | Example |
| --- | --- | --- |
| kp <value> | Kp | kp 2.5 |
| ki <value> | Ki | ki 0.00 |
| kd <value> | Kd | kd 30 |
| bsr <value> | Base speed right | bsr 110 |
| bsl <value> | Base speed left | bsl 110 |
| msr <value> | Max speed right | msr 130 |
| msl <value> | Max speed left | msl 130 |

# 10. Startup and Calibration Sequence

Initialize the QTR sensor type as RC and assign all sensor pins.

Start Serial and Bluetooth communication.

Set motor and button pins.

Brake both motors initially for safety.

Run calibration by rotating the robot in both directions so the sensor array learns dark and light reflectance levels.

# 11. How to Use

Power the robot using the battery and voltage regulator.

Place the robot on the track after calibration is complete.

Open a Bluetooth terminal app and connect to ESP32-B.

Send tuning commands and observe the response in real time.

Increase or reduce Kp, Kd, and speed values until the robot tracks cleanly at the desired speed.

# 12. Important Code Notes

The code defines BASE_SPEED_L as 131100, which looks unusually large for a motor speed variable. It likely needs verification before final use.

The loop() function continuously calls pid_control(), so the robot always tries to stay on the line unless power is removed or logic is extended.

Buttons are declared in the code, but no button-based behavior is currently implemented in loop().

# 13. Future Improvements

Add lost-line recovery logic when the line is no longer detected.

Store tuned PID values in EEPROM or flash memory.

Add a start/stop switch and button-based mode selection.

Display live status values on OLED or LCD.

Add telemetry such as sensor position, error, and motor outputs.

# 14. Conclusion

This fast line follower bot combines a high-speed motor platform, a multi-sensor QTR array, PID control, and live Bluetooth tuning to achieve responsive line tracking. The design is suitable as a competition-style robot and also works well as a strong embedded systems project for documentation, demonstration, and further development.